Gesture Recognition Data Collection Workflow

MATLAB UI + Arduino + HC-05 Bluetooth + CSV Dataset

Purpose: This document explains the final workflow for collecting gesture data. MATLAB shows the gesture image and labels the data. Arduino only calculates the selected data type and sends readings through HC-05 Bluetooth.

# 1. System Overview

| Component | Role |
| --- | --- |
| MATLAB UI | Shows gesture image, selects data type, receives readings, labels data, saves CSV. |
| Python scanner | Finds the correct HC-05 Bluetooth COM port and saves it to a text file. |
| Arduino | Receives TYPE command, uses selected calculation method, sends DATA,value lines. |
| HC-05 Bluetooth | Wireless serial link between Arduino and MATLAB. |
| CSV file | Stores DataType, GestureNumber, GestureName, Iteration, SampleNumber, Voltage, and Time. |

## Overall Flow Diagram

MATLAB shows gesture image ↓ User makes the shown gesture ↓ MATLAB sends TYPE,x to Arduino ↓ Arduino confirms TYPE_SET,x ↓ User clicks Start ↓ Arduino sends 10 × 40 readings as DATA,value ↓ MATLAB labels readings using current image ↓ MATLAB saves or updates the selected CSV file

# 2. Final Operating Workflow

| Step | Action | What happens |
| --- | --- | --- |
| 1 | Open MATLAB UI | Run GestureCollectorUI_Final_TypeRequiredBeforeStart or the latest UI file. |
| 2 | Scan HC-05 | MATLAB runs the Python scanner and reads last_hc05_port.txt. |
| 3 | Connect HC-05 | MATLAB opens the detected COM port and sends HELLO. |
| 4 | Select Type | Choose Type 1, Type 2, Type 3, or Type 4 in MATLAB. |
| 5 | Set Data Type | MATLAB sends TYPE,x to Arduino. Arduino replies TYPE_SET,x. |
| 6 | Show Gesture Image | MATLAB displays the current gesture, for example A. |
| 7 | User Makes Gesture | User performs the hand sign shown on the MATLAB screen. |
| 8 | Start Recording | MATLAB sends N. Arduino starts the selected calculation method. |
| 9 | Receive Readings | Arduino sends 400 readings as DATA,value. |
| 10 | Save CSV | MATLAB labels, counts, timestamps, and saves the data. |

# 3. Data Type Selection

In this project, the data type is not just a CSV label. It controls the calculation method used inside Arduino.

| MATLAB selection | Command sent | Arduino action |
| --- | --- | --- |
| Type 1 - Method 1 | TYPE,1 | Use calculation method 1 |
| Type 2 - Method 2 | TYPE,2 | Use calculation method 2 |
| Type 3 - Method 3 | TYPE,3 | Use calculation method 3 |
| Type 4 - Method 4 | TYPE,4 | Use calculation method 4 |

Important: The Start button should stay disabled until Arduino replies with TYPE_SET,x. This confirms that Arduino has selected the correct calculation method before recording begins.

# 4. Communication Protocol

| Command / Message | Direction | Meaning |
| --- | --- | --- |
| HELLO | MATLAB/Python → Arduino | Check whether the port is HC-05. |
| HC05_GESTURE_DEVICE | Arduino → MATLAB/Python | Correct Bluetooth device found. |
| USB_PORT_IGNORE | Arduino USB → MATLAB/Python | Ignore Arduino USB port during scan. |
| TYPE,1 to TYPE,4 | MATLAB → Arduino | Select Arduino calculation method. |
| TYPE_SET,1 to TYPE_SET,4 | Arduino → MATLAB | Arduino confirmed selected method. |
| N or START | MATLAB → Arduino | Start recording 10 × 40 readings. |
| DATA,value | Arduino → MATLAB | One calculated reading. |
| END | Arduino → MATLAB | Current gesture recording is finished. |
| RESET | MATLAB → Arduino | Reset Arduino and UI state. |

# 5. How MATLAB Decides Iteration and Sample Number

Arduino sends only readings. MATLAB counts the received DATA lines and assigns iteration and sample number automatically.

1 gesture recording = 10 iterations × 40 readings = 400 readings Reading 1 to 40 → Iteration 1 Reading 41 to 80 → Iteration 2 Reading 81 to 120 → Iteration 3 ... Reading 361 to 400 → Iteration 10

# 6. CSV File Structure

The CSV stores all required labels and readings. MATLAB adds the gesture label from the image shown on screen.

| CSV Column | Meaning |
| --- | --- |
| DataType | Selected Arduino calculation method, 1 to 4. |
| GestureNumber | Gesture class number from MATLAB UI, for example 1 for A. |
| GestureName | Gesture label from current MATLAB image, for example A. |
| Iteration | Calculated by MATLAB from reading count. |
| SampleNumber | Calculated by MATLAB from reading count, 1 to 40. |
| Voltage | Calculated value sent by Arduino. |
| Time | MATLAB receive time, saved as Excel-safe text. |

Example CSV row: 1, 1, A, 1, 1, 1.398, 2026-06-05_22-04-03.456

# 7. Re-recording and Updating Existing CSV Data

If the same Gesture + Type is recorded again, MATLAB should replace only that matching old data and keep all other saved data safe.

Before: A Type 1 = 400 rows B Type 1 = 400 rows Record A Type 1 again: Old A Type 1 rows are removed New A Type 1 rows are added B Type 1 rows stay unchanged

Result: Re-recording A Type 1 does not create duplicate A rows and does not delete B. It only replaces the same Gesture + Type.

# 8. Button Meanings in MATLAB UI

| Button | Use |
| --- | --- |
| Scan HC-05 | Runs Python scanner and finds the HC-05 Bluetooth COM port. |
| Connect HC-05 | Connects MATLAB to the detected HC-05 port. |
| Disconnect | Closes the Bluetooth serial connection. |
| Set Data Type | Sends TYPE,x to Arduino and waits for TYPE_SET,x. |
| Start | Starts recording only after Arduino confirms the selected type. |
| Set CSV Name / New File | Selects or creates the CSV file to save/update. |
| Jump to Selected Gesture | Changes the MATLAB image/label to a selected gesture. |
| Clear MATLAB Memory | Clears temporary MATLAB session data, not the saved CSV file. |
| Reset Arduino + UI | Sends RESET to Arduino and resets MATLAB UI state. |

# 9. Recommended Folder Structure

D:\data │ ├── CSV │ └── gesture_dataset.csv │ ├── gestures │ ├── A_sign.png │ ├── B_sign.png │ └── ... │ └── HC05Scanner ├── scan_hc05.py └── last_hc05_port.txt

# 10. Final Short Summary

Scan HC-05 ↓ Connect HC-05 ↓ Select Type ↓ Set Data Type and wait for TYPE_SET ↓ MATLAB shows gesture image ↓ User makes gesture ↓ Click Start ↓ Arduino sends DATA,value readings ↓ MATLAB labels and saves to CSV